Steel Coil Planner Pro

Functional Specification - Draft v1.0

# 1. Purpose

Steel Coil Planner Pro is a cargo planning workspace for steel coil stowage. The application must help the user select a vessel and holds, import or simulate cargo, allocate cargo by hold length, and display consistent Top View and Cross Section based on one shared geometry model.

# 2. Core Operating Rules

All holds for the selected vessel must be visible in the Allocation Workspace at the same time.

Each hold has its own independent drag marker from 0 m to maximum hold length.

The marker must snap to complete blocks, but it must also be able to go to 0 m cargo.

Moving a marker updates hold length, blocks, coils, weight, remaining cargo, and summary live.

The user must not need to press Generate Plan after every marker movement.

Bottom Row can be Auto or Manual.

Upper Row remains Manual by default and must not be silently changed by the algorithm.

Wedge can be Auto or Manual.

Top View and Cross Section must use the same geometry model, never separate calculations.

# 3. Main Modules

| Module | Responsibility |
| --- | --- |
| Fleet Library | Store vessels and multiple holds per vessel. Save and reload all hold dimensions and planning defaults. |
| Cargo Source | Use Cargo List mode when a list exists; use Simulation mode when no list exists. |
| Allocation Workspace | Display all holds, drag markers, live allocations, and summaries. |
| Width Arrangement Engine | Calculate Bottom, Upper, Wedge, central gap, and coil coordinates across hold width. |
| Renderer | Draw Top View and Cross Section from the same coordinate output. |
| Reports | Future module for export of cargo plan, summaries, and stowage report. |

# 4. Cargo Source Modes

## 4.1 Cargo List Mode

The user imports CSV or Excel. The app detects coil ID, weight, width, and diameter where available. If diameter is missing, the user enters Average Diameter.

## 4.2 Simulation Mode

The user can simulate a cargo without a cargo list. Required inputs: coil width, diameter, weight, row gap, bottom mode, upper values, wedge mode, and either total coils or total tonnes.

# 5. Allocation Workspace

All vessel holds are displayed on the same page. The hold dropdown only moves focus to the selected hold; it must not reload or erase any allocation. Each hold card displays its own marker and live values.

| Displayed value per hold | Description |
| --- | --- |
| Used length | Length selected by marker or manual value. |
| Blocks | Complete blocks fitting within used length. |
| Coils | Coils allocated to that hold. |
| Weight | Estimated or actual weight allocated to that hold. |
| Status | OK, empty, exceeds hold capacity, or remaining cargo not allocated. |

# 6. Summary Panel

The right side summary must always show total voyage cargo and a per-hold breakdown: length, blocks, coils, weight, and percent of total cargo. Remaining cargo must be shown as coils, tonnes, and estimated required length.

# 7. Rendering Rules

Cross Section is the geometric reference view.

Bottom coils sit on tank top.

Upper coils sit between two bottom coils and touch them.

Wedge coils sit only in real gaps generated by the engine and must never overlap.

Top View must reflect the same block/tier result as Cross Section, with longitudinal stowage representation.

# 8. Next Acceptance Tests

Create a vessel with Hold 1 and Hold 2, save, reload, and confirm all dimensions remain unchanged.

Switch hold dropdown and confirm the page only focuses the hold, without changing allocation.

Move Hold 1 marker to 0 m and confirm all cargo moves to following holds.

Use Simulation Mode with no cargo list and verify summary is calculated.

Set Bottom Manual and Upper Manual and verify Cross Section follows manual values.

Confirm Top View and Cross Section represent the same engine output.